dbus-mqtt-devices

Venus OS Integration Guide

Propagate Home Assistant sensors to Victron VRM

# Overview

dbus-mqtt-devices (by freakent) is a Venus OS driver that lets any MQTT-capable device — including Home Assistant — self-register on the Victron D-Bus and publish live data to VRM. Unlike the mr-manuel single-sensor drivers, this driver supports multiple devices and sensor types simultaneously using a compact registration protocol.

Supported D-Bus services:

- temperature (com.victronenergy.temperature.*)

- tank (com.victronenergy.tank.*)

- pvinverter (com.victronenergy.pvinverter.*)

- grid (com.victronenergy.grid.*)

- gps (com.victronenergy.gps.*)

- evcharger (com.victronenergy.evcharger.*)

- battery — JK BMS (com.victronenergy.battery.*)

- vebus (com.victronenergy.vebus.*)

⚠️ Not supported on CCGX Installation on CCGX (Color Control GX) can cause random reboots due to limited resources. Use only on Cerbo GX, Ekrano GX, or a Raspberry Pi running Venus OS.

# Architecture

The data flow from Home Assistant to VRM involves three stages:

| Stage | Description |
| --- | --- |
| 1. Publish | HA publishes a JSON MQTT message to the Cerbo's built-in broker (or an external one). |
| 2. Register | HA (or any client) sends a registration payload to device/<clientId>/Status — telling the driver what sensor type to create. |
| 3. D-Bus → VRM | The driver registers the sensor on the D-Bus. Victron VRM picks it up automatically on the next poll. |

# Prerequisites

## 1. Enable MQTT on the Cerbo GX

On the Cerbo GX touch display or Remote Console:

- Go to Settings → Services

- Enable MQTT on LAN (Plaintext) — port 1883

- Optionally enable MQTT on LAN (SSL) — port 8883

💡 Tip If you already run Mosquitto in Home Assistant and want to bridge it to the Cerbo, you can — but for simplicity these instructions use the Cerbo's own broker directly.

## 2. Enable SSH Root Access on the Cerbo GX

- Go to Settings → General

- Set Access Level to Superuser

- Note the IP address of the Cerbo GX (shown in Settings → Ethernet or Wi-Fi)

## 3. Verify MQTT broker is reachable (optional)

From any device on the same network, test connectivity:

# Subscribe to all Victron topics to verify broker is alive

mosquitto_sub -h <cerbo-ip> -p 1883 -t '#' -v

# Installation on Venus OS

## Step 1 — SSH into the Cerbo GX

ssh root@<cerbo-ip>

# Accept the host key fingerprint when prompted

## Step 2 — Download and extract the driver

mkdir -p /data/drivers

cd /data/drivers

wget -O dbus-mqtt-devices.zip \

https://github.com/freakent/dbus-mqtt-devices/archive/refs/tags/v0.9.0.zip

unzip dbus-mqtt-devices.zip

📌 Version note v0.9.0 is required for Venus OS 3.60+. Always check https://github.com/freakent/dbus-mqtt-devices/releases for the latest version tag.

## Step 3 — Run the setup script

cd /data/drivers/dbus-mqtt-devices-0.9.0

./bin/setup.sh

The script installs Python dependencies and creates the rc.local entry.

## Step 4 — Verify autostart entry

cat /data/rc.local

# Expected output:

/data/drivers/dbus-mqtt-devices-0.9.0/bin/setup-dependencies.sh

ln -s /data/drivers/dbus-mqtt-devices-0.9.0/bin/service /service/dbus-mqtt-devices

## Step 5 — Reboot the Cerbo GX

reboot

The driver starts automatically on every subsequent boot. It also self-updates its Python dependencies after each Venus OS firmware upgrade.

# The Registration Protocol

This is the core concept. Every time Home Assistant connects to MQTT (or on HA startup), it must send a registration payload to tell the driver which sensor types to create.

## Step A — HA subscribes to the DBus response topic

Home Assistant must subscribe to receive the VRM device instance numbers back from the driver:

device/<clientId>/DBus

⚠️ Important The <clientId> must be a short unique string. Avoid special characters: , . - / :

## Step B — HA publishes the Status registration payload

Topic: device/<clientId>/Status

Payload (single temperature sensor):

{

"clientId": "ha_sensors",

"connected": 1,

"version": "HA v2024.12",

"services": {

"t1": "temperature"

}

}

Multiple sensor types example:

Payload (two temperatures + tank):

{

"clientId": "ha_sensors",

"connected": 1,

"version": "HA v2024.12",

"services": {

"t1": "temperature",

"t2": "temperature",

"tk1": "tank"

}

}

## Step C — Driver responds with D-Bus instance numbers

The driver replies on device/<clientId>/DBus with the VRM instance numbers needed for publishing data:

{

"clientId": "ha_sensors",

"portalId": "a1b2c3d4e5f6",

"deviceInstance": {

"t1": 24,

"t2": 25,

"tk1": 20

}

}

📌 Save these instance numbers HA must store the returned deviceInstance values. They are used to construct the W/<portalId>/... write topics in the next step.

## Step D — HA publishes sensor values

Once registered, publish actual readings using the Victron dbus-mqtt write topic format:

# Temperature value

Topic: W/<portalId>/temperature/<deviceInstance>/Temperature

Payload: {"value": 21.5}

# Humidity (optional, same sensor)

Topic: W/<portalId>/temperature/<deviceInstance>/Humidity

Payload: {"value": 58.0}

# Custom name (optional)

Topic: W/<portalId>/temperature/<deviceInstance>/CustomName

Payload: {"value": "Living Room"}

# Home Assistant Configuration

## MQTT Integration

Ensure the MQTT integration is configured in HA pointing at the Cerbo GX's IP on port 1883:

# configuration.yaml

mqtt:

broker: <cerbo-ip>

port: 1883

client_id: ha_sensors

💡 Tip Use the same client_id here as the clientId in the registration payload so the MQTT session and D-Bus registration stay in sync.

## Automation: Registration on HA Start

This automation fires on HA startup and sends the device registration:

automation:

- alias: Register sensors with Victron

trigger:

- platform: homeassistant

event: start

action:

- service: mqtt.publish

data:

topic: device/ha_sensors/Status

retain: true

payload: >

{"clientId":"ha_sensors","connected":1,

"version":"HA v2024.12",

"services":{"t1":"temperature","t2":"temperature"}}

## Automation: Capture D-Bus Instance Numbers

Store the returned instance numbers as input_number helpers or in the automation context:

automation:

- alias: Store Victron device instances

trigger:

- platform: mqtt

topic: device/ha_sensors/DBus

action:

- service: input_text.set_value

target:

entity_id: input_text.victron_dbus_response

data:

value: '{{ trigger.payload }}'

## Automation: Publish Temperature Values

Replace <portalId> and <instance> with the values received in the DBus response:

automation:

- alias: Push living room temp to Victron

trigger:

- platform: state

entity_id: sensor.living_room_temperature

action:

- service: mqtt.publish

data_template:

topic: >

W/<portalId>/temperature/<instance_t1>/Temperature

payload: >

{"value": {{ states('sensor.living_room_temperature') ¦ float(0) }}}

# Troubleshooting

## Check driver is running

# Check service status

svstat /service/dbus-mqtt-devices

# Watch live driver log

tail -f /var/log/dbus-mqtt-devices/current ¦ tai64nlocal

## Monitor MQTT traffic on Cerbo

# Subscribe to device registration topics

mosquitto_sub -h 127.0.0.1 -p 1883 -t 'device/#' -v

# Subscribe to all write topics

mosquitto_sub -h 127.0.0.1 -p 1883 -t 'W/#' -v

## Common Issues

| Problem | Solution |
| --- | --- |
| ModuleNotFoundError: dataclasses | Run setup-dependencies.sh manually before setup.sh |
| Numeric values sent as strings | Check JSON payloads — values must NOT be quoted: {"value": 21.5} not {"value": "21.5"} |
| Sensor appears then disappears | Set retain: true on the Status registration payload so it persists across Cerbo reboots |
| NaN / null / infinity errors | Validate your payload in HA — JavaScript-generated payloads sometimes produce these for undefined sensors |
| Sensor not visible in VRM | Allow up to 5 minutes for VRM to refresh. Check D-Bus registration with dbus-spy on the Cerbo |
| Driver not autostarting after firmware update | Venus OS upgrades are handled automatically, but run setup.sh again if the symlink is broken |

# Last Will Topic (LWT)

To automatically disconnect devices from D-Bus when they go offline, configure an MQTT Last Will:

# In your MQTT client config, set:

LWT topic: device/ha_sensors/Status

LWT payload: {"clientId":"ha_sensors","connected":0,"version":"HA v2024.12","services":{}}

LWT retain: true

When HA disconnects unexpectedly, the broker delivers the LWT payload automatically, and the driver removes the device from D-Bus — keeping VRM clean.

# Quick Reference: Topic Structure

| Topic | Purpose |
| --- | --- |
| device/<clientId>/Status | HA → Cerbo: registration payload (retain: true) |
| device/<clientId>/DBus | Cerbo → HA: VRM instance numbers response |
| W/<portalId>/temperature/<instance>/Temperature | HA → Cerbo: live temperature value (°C) |
| W/<portalId>/temperature/<instance>/Humidity | HA → Cerbo: live humidity value (%) |
| W/<portalId>/temperature/<instance>/Pressure | HA → Cerbo: live pressure value (hPa) |
| W/<portalId>/temperature/<instance>/CustomName | HA → Cerbo: display name in VRM |
| W/<portalId>/tank/<instance>/Level | HA → Cerbo: tank fill level (0–100 %) |
| W/<portalId>/tank/<instance>/Remaining | HA → Cerbo: remaining volume (m³) |

# References

- Driver source code: https://github.com/freakent/dbus-mqtt-devices

- Victron D-Bus service documentation: https://github.com/victronenergy/venus/wiki/dbus

- Venus OS root access guide: https://www.victronenergy.com/live/ccgx:root_access

- Victron VRM portal: https://vrm.victronenergy.com