EV Charger Integration Guide

Feyree  ↔  Home Assistant  ↔  Victron Venus OS  ↔  VRM

Solar surplus charging  ·  3-phase power  ·  Bidirectional ESS control

Portal ID: <venus_id>  ·  MQTT prefix: venus-home/

dbus-mqtt-devices (freakent)  ·  Venus OS Large  ·  Node-RED

# 1. Overview

This guide documents the complete integration of a Feyree 3-phase EV charger (Tuya-based) with the Victron energy management ecosystem, exposing it in VRM as a native EV charger service and enabling automatic solar surplus charging via Node-RED.

| Component | Role |
| --- | --- |
| Home Assistant | Reads Feyree data via tuya-local, publishes to MQTT, applies Victron commands |
| Mosquitto (HA addon) | MQTT bridge between HA broker and Cerbo GX broker (prefix: venus-home/) |
| Cerbo GX — Venus OS Large | Runs dbus-mqtt-devices driver and Node-RED surplus flow |
| dbus-mqtt-devices (freakent) | Registers virtual EV charger service on D-Bus → visible in VRM |
| Node-RED (on Cerbo) | Reads Victron data, calculates surplus, writes SetCurrent/StartStop |
| VRM | Displays EV charger, allows mode switching (Manual / Auto / Scheduled) |

## 1.1  Architecture

The data flows in two directions simultaneously:

PUSH (HA → VRM):

Feyree (tuya-local) → HA entities → MQTT publish

→ venus-home/W/<portalId>/evcharger/<instance>/...

→ dbus-mqtt-devices → D-Bus → VRM

PULL (Victron → Feyree):

VRM mode change / ESS SetCurrent

→ N/<portalId>/evcharger/<instance>/SetCurrent

→ venus-home/N/... → HA MQTT trigger

→ select.ev_charger_set_charge_current (tuya-local)

AUTO (Node-RED surplus loop, every 30s):

Victron D-Bus nodes (Grid L1/L2/L3, SOC, Batt Power)

+ EV Ac/Power + EV Status + EV Mode (from Victron)

→ fn-calc → W/.../SetCurrent + W/.../StartStop

## 1.2  Prerequisites

- Cerbo GX running Venus OS Large (≥ v3.60)

- dbus-mqtt-devices v0.9.0 installed on Cerbo

- Node-RED enabled on Cerbo (Settings → Venus OS Large Features)

- Home Assistant with Mosquitto broker add-on

- Feyree charger added to Tuya/SmartLife app

- tuya-local HACS integration installed in HA

- MQTT on LAN (Plaintext) enabled on Cerbo GX (Settings → Services)

ℹ Portal ID Your Victron portal ID is <venus_id>. It appears in all MQTT topic paths throughout this guide.

# 2. Mosquitto Bridge Configuration

The bridge connects HA's Mosquitto broker to the Cerbo's built-in broker. The topic prefix venus-home/ is stripped automatically in both directions by the bridge rule.

## 2.1  Bridge config file

/share/mosquitto/victron-bridge.conf (create if not present):

connection cerbo-bridge

address <cerbo-ip>:1883

bridge_protocol_version mqttv311

cleansession true

try_private false

start_type automatic

# All topics — single rule handles both directions

# venus-home/ prefix is stripped on the Cerbo side

topic # both 0 venus-home/

## 2.2  How the prefix mapping works

| HA publishes | Cerbo receives |
| --- | --- |
| venus-home/device/ha_bridge/Status | device/ha_bridge/Status |
| venus-home/W/<venus_id>/evcharger/<n>/SetCurrent | W/<venus_id>/evcharger/<n>/SetCurrent |
| (Cerbo publishes) N/<venus_id>/evcharger/<n>/Mode | venus-home/N/<venus_id>/evcharger/<n>/Mode |

⚠ Restart required Restart the Mosquitto add-on in HA after saving the bridge config file.

# 3. Home Assistant Setup

## 3.1  Feyree via tuya-local

Install tuya-local from HACS. With the Feyree charger added to the Tuya/SmartLife app, tuya-local auto-detects it and creates all entities. The key entities used in this integration are:

| HA Entity | Type | Description |
| --- | --- | --- |
| sensor.ev_charger_status | sensor | Charger state (idle / connected / charging / charged) |
| sensor.ev_charger_power | sensor | Total AC power drawn (W) |
| sensor.ev_charger_current | sensor | L1 current (A) |
| sensor.ev_charger_current_b | sensor | L2 current (A) |
| sensor.ev_charger_current_c | sensor | L3 current (A) |
| sensor.ev_charger_voltage | sensor | L1 voltage (V) |
| sensor.ev_charger_voltage_b | sensor | L2 voltage (V) |
| sensor.ev_charger_voltage_c | sensor | L3 voltage (V) |
| sensor.ev_charger_energy | sensor | Total energy charged (kWh) |
| number.ev_charger_set_charge_current | number | Current setpoint (A) — writable |
| select.ev_charger_charging | select | Start/stop control: On = charging, Off = stopped |

ℹ Per-phase power Feyree does not expose per-phase power directly. This integration derives it as V × I for each phase, which is accurate for EV charging (power factor ≈ 1.0).

## 3.2  Input Text Helpers

Create these helpers in HA (Settings → Devices & Services → Helpers → Add Helper → Text):

input_text.victron_instance_t1

input_text.victron_instance_t2

input_text.victron_instance_t3

input_text.victron_instance_t4

input_text.victron_instance_ev1

# 4. Home Assistant Automations

All MQTT topics use the venus-home/ prefix because the bridge strips it before forwarding to the Cerbo.

## 4.1  Registration

Fires on HA startup and every hour as a keepalive. The payload registers both temperature sensors and the EV charger with dbus-mqtt-devices in a single message.

alias: "Victron - Register devices"

trigger:

- platform: homeassistant

event: start

- platform: time_pattern

hours: "/1"

action:

- service: mqtt.publish

data:

topic: "venus-home/device/ha_bridge/Status"

retain: true

payload: >

{"clientId":"ha_bridge","connected":1,

"version":"HA-bridge v1.0",

"services":{

"t1":"temperature","t2":"temperature",

"t3":"temperature","t4":"temperature",

"ev1":"evcharger"}}

## 4.2  Capture D-Bus Instance Numbers

dbus-mqtt-devices responds on device/ha_bridge/DBus with numeric VRM instance numbers. This automation stores all five values into helpers for use in all other automations.

alias: "Victron - Store D-Bus instances"

trigger:

- platform: mqtt

topic: "venus-home/device/ha_bridge/DBus"

action:

- service: input_text.set_value

target:

entity_id: input_text.victron_instance_t1

data:

value: "{{ trigger.payload_json.deviceInstance.t1 }}"

- service: input_text.set_value

target:

entity_id: input_text.victron_instance_t2

data:

value: "{{ trigger.payload_json.deviceInstance.t2 }}"

- service: input_text.set_value

target:

entity_id: input_text.victron_instance_t3

data:

value: "{{ trigger.payload_json.deviceInstance.t3 }}"

- service: input_text.set_value

target:

entity_id: input_text.victron_instance_t4

data:

value: "{{ trigger.payload_json.deviceInstance.t4 }}"

- service: input_text.set_value

target:

entity_id: input_text.victron_instance_ev1

data:

value: "{{ trigger.payload_json.deviceInstance.ev1 }}"

## 4.3  Push EV Charger Data to VRM

Fires on any state change of the Feyree entities. Publishes power, per-phase values (derived as V×I), status, current setpoint, and energy to the virtual EV charger service.

alias: "Victron - Push EV charger"

trigger:

- platform: state

entity_id:

- sensor.ev_charger_status

- sensor.ev_charger_power

- sensor.ev_charger_current

- sensor.ev_charger_current_b

- sensor.ev_charger_current_c

- sensor.ev_charger_voltage

- sensor.ev_charger_voltage_b

- sensor.ev_charger_voltage_c

- number.ev_charger_set_charge_current

- sensor.ev_charger_energy

- select.ev_charger_charging

condition:

- condition: template

value_template: "{{ states('input_text.victron_instance_ev1') != '' }}"

variables:

status_map:

idle: 0

standby: 0

connected: 1

charging: 2

charged: 3

waiting: 6

fault: 11

base: >

venus-home/W/<venus_id>/evcharger/

{{- states('input_text.victron_instance_ev1') -}}

action:

# Status

- service: mqtt.publish

data:

topic: "{{ base }}/Status"

payload: >

{"value": {{ status_map.get(states('sensor.ev_charger_status'), 0) }}}

# StartStop

- service: mqtt.publish

data:

topic: "{{ base }}/StartStop"

payload: >

{"value": {{ 1 if states('select.ev_charger_charging') == 'On' else 0 }}}

# Total power

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/Power"

payload: '{"value": {{ states("sensor.ev_charger_power") ¦ float(0) }}}'

# L1 — power from V×I

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L1/Power"

payload: >

{"value": {{ ((states("sensor.ev_charger_voltage") ¦ float(0))

* (states("sensor.ev_charger_current") ¦ float(0))) ¦ round(1) }}}

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L1/Voltage"

payload: '{"value": {{ states("sensor.ev_charger_voltage") ¦ float(0) }}}'

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L1/Current"

payload: '{"value": {{ states("sensor.ev_charger_current") ¦ float(0) }}}'

# L2

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L2/Power"

payload: >

{"value": {{ ((states("sensor.ev_charger_voltage_b") ¦ float(0))

* (states("sensor.ev_charger_current_b") ¦ float(0))) ¦ round(1) }}}

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L2/Voltage"

payload: '{"value": {{ states("sensor.ev_charger_voltage_b") ¦ float(0) }}}'

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L2/Current"

payload: '{"value": {{ states("sensor.ev_charger_current_b") ¦ float(0) }}}'

# L3

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L3/Power"

payload: >

{"value": {{ ((states("sensor.ev_charger_voltage_c") ¦ float(0))

* (states("sensor.ev_charger_current_c") ¦ float(0))) ¦ round(1) }}}

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L3/Voltage"

payload: '{"value": {{ states("sensor.ev_charger_voltage_c") ¦ float(0) }}}'

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/L3/Current"

payload: '{"value": {{ states("sensor.ev_charger_current_c") ¦ float(0) }}}'

# Current setpoint

- service: mqtt.publish

data:

topic: "{{ base }}/Current"

payload: '{"value": {{ states("number.ev_charger_set_charge_current") ¦ float(0) }}}'

# Hardware maximum

- service: mqtt.publish

data:

topic: "{{ base }}/MaxCurrent"

payload: '{"value": 32}'

# Total energy

- service: mqtt.publish

data:

topic: "{{ base }}/Ac/Energy/Forward"

payload: '{"value": {{ states("sensor.ev_charger_energy") ¦ float(0) }}}'

# Display name

- service: mqtt.publish

data:

topic: "{{ base }}/CustomName"

payload: '{"value": "ev_charger"}'

## 4.4  Bidirectional: Apply Victron Commands to Feyree

When Victron ESS or the Node-RED surplus flow writes SetCurrent or StartStop, these are published on the N/ (read) topic and forwarded by the bridge to HA. This automation catches them and applies them to the Feyree.

alias: "Victron - Apply EV charger commands from Victron"

trigger:

- platform: mqtt

topic: "venus-home/N/<venus_id>/evcharger/+/SetCurrent"

id: set_current

- platform: mqtt

topic: "venus-home/N/<venus_id>/evcharger/+/StartStop"

id: start_stop

condition:

# Confirm message is for our registered EV instance

- condition: template

value_template: >

{{ trigger.topic.split('/')[4] ==

states('input_text.victron_instance_ev1') }}

action:

- choose:

# SetCurrent — clamp between 6A and 32A

- conditions:

- condition: trigger

id: set_current

sequence:

- service: number.set_value

target:

entity_id: number.ev_charger_set_charge_current

data:

value: >

{{ [ [trigger.payload_json.value ¦ int, 6] ¦ max, 32 ] ¦ min }}

# StartStop — map 1/0 to On/Off select option

- conditions:

- condition: trigger

id: start_stop

sequence:

- choose:

- conditions:

- condition: template

value_template: "{{ trigger.payload_json.value == 1 }}"

sequence:

- service: select.select_option

target:

entity_id: select.ev_charger_charging

data:

option: "On"

- conditions:

- condition: template

value_template: "{{ trigger.payload_json.value == 0 }}"

sequence:

- service: select.select_option

target:

entity_id: select.ev_charger_charging

data:

option: "Off"

## 4.5  Disconnect on HA Shutdown

alias: "Victron - Disconnect on shutdown"

trigger:

- platform: homeassistant

event: shutdown

action:

- service: mqtt.publish

data:

topic: "venus-home/device/ha_bridge/Status"

retain: true

payload: >

{"clientId":"ha_bridge","connected":0,

"version":"HA-bridge v1.0","services":{}}

# 5. Node-RED Surplus Charging Flow

The Node-RED flow runs on the Cerbo GX and handles automatic solar surplus charging. It only activates when the Victron mode is set to Auto (value 1) in VRM. It reads live system data from the Victron D-Bus, calculates the available surplus power, and commands the Feyree to charge at the optimal current every 30 seconds.

## 5.1  Flow sections

| Section | Nodes | Purpose |
| --- | --- | --- |
| 1 — Config | inject + function | Set all parameters in one place |
| 2 — Instance | mqtt in + function | Capture EV D-Bus instance from dbus-mqtt-devices |
| 3 — Data | 5× Victron input + 2× mqtt in | Continuous update of grid, battery, EV data |
| 4 — Calculation | inject (30s) + function + 2× mqtt out | Surplus algorithm + ramp + publish |

## 5.2  Configuration parameters

All parameters are set in the CONFIG function node (Section 1). Edit these values for your system:

| Parameter | Default | Description |
| --- | --- | --- |
| PORTAL_ID | <venus_id> | Victron VRM portal ID — do not change |
| MIN_SOC | 20 | Battery % — EV stops charging below this level |
| MIN_CURRENT | 6 | Minimum A Feyree accepts (hardware limit) |
| MAX_CURRENT | 32 | Maximum A for your Feyree model |
| PHASES | 3 | Number of phases (3 for 3-phase Feyree) |
| VOLTAGE | 230 | Volts per phase |
| MAX_RAMP_UP | 6 | Maximum A increase per 30s step (prevents charger errors) |
| MAX_RAMP_DOWN | 2 | Maximum A decrease per 30s step (Feyree limitation) |
| GRID_SETPOINT | 0 | Target grid power (W). Set to -200 to keep 200W export margin |
| MAX_BATT_DISCHARGE | 500 | Max W battery can discharge to supplement EV — prevents battery drain |
| AUTO_MODE_VALUE | 1 | Victron mode value for Auto — do not change |

## 5.3  Algorithm logic

Every 30 seconds the calculation node runs through these guards in order. The first failure exits immediately with a descriptive node status:

- EV instance known? (from dbus-mqtt-devices DBus response)

- Battery SOC data received from D-Bus?

- Victron mode = Auto (1)? If Manual (0) or Scheduled (2) → inactive

- Car connected? (EV status ≠ 0)

- Car not fully charged? (EV status ≠ 3)

- Battery SOC ≥ MIN_SOC? If not → stop charger, protect battery

- Battery not discharging beyond MAX_BATT_DISCHARGE? If yes → hold/reduce

If all guards pass, the surplus is calculated:

available_watts = ev_power - grid_power + GRID_SETPOINT

target_current = available_watts / (PHASES × VOLTAGE)

# Apply ramp limits

if target_current ≥ prev_current:

new_current = min(prev_current + MAX_RAMP_UP, target_current, MAX_CURRENT)

else:

new_current = max(prev_current - MAX_RAMP_DOWN, target_current, MIN_CURRENT)

# Round to integer and check minimum

new_current = round(new_current)

should_charge = new_current ≥ MIN_CURRENT

# Publish

W/<portalId>/evcharger/<instance>/SetCurrent ← new_current

W/<portalId>/evcharger/<instance>/StartStop ← 1 or 0

✓ Node status The fn-calc node displays live state directly in Node-RED without needing the debug panel: e.g. 16A / 11.0kW ¦ surplus:3680W ¦ SOC:74% ¦ grid:-220W

## 5.4  Mode gate

The EV Mode is read directly from Victron's MQTT broker (N/<venus_id>/evcharger/+/Mode). Victron publishes these values:

| Value | Victron Mode | Flow behaviour |
| --- | --- | --- |
| 0 | Manual | fn-calc exits: Manual mode — inactive |
| 1 | Auto | Flow runs normally |
| 2 | Scheduled | fn-calc exits: Scheduled mode — inactive |

When you change the mode in VRM, the Cerbo publishes the new value immediately and Node-RED picks it up within a second.

## 5.5  Importing the flow

- Open Node-RED at https://venus.local:1881/

- Click hamburger menu → Import

- Select the file ev-surplus-flow.json

- Click Import

- Open the ⚙️ CONFIG node and verify/adjust parameters

- Open all five Victron input nodes (Grid L1/L2/L3, Battery SOC, Battery Power) and confirm the service and measurement dropdowns show correct values

- Click Deploy

- Watch the fn-ev-mode node status — it should show Auto, Manual or Scheduled

- Watch fn-calc status after the first 30s tick

# 6. MQTT Topic Reference

All topics shown as HA sees them (with venus-home/ prefix). On the Cerbo the prefix is absent.

## 6.1  Registration and control

| Topic (as seen by HA) | Direction | Purpose |
| --- | --- | --- |
| venus-home/device/ha_bridge/Status | HA → Cerbo | Device registration payload (retain: true) |
| venus-home/device/ha_bridge/DBus | Cerbo → HA | Instance numbers response from dbus-mqtt-devices |

## 6.2  EV charger push topics (HA → VRM)

| Topic suffix | Payload | Description |
| --- | --- | --- |
| /Status | { value: 0-11 } | 0=disconnected 1=connected 2=charging 3=charged 6=waiting 11=fault |
| /StartStop | { value: 0¦1 } | 1=start 0=stop |
| /Ac/Power | { value: W } | Total AC power (W) |
| /Ac/L1/Power | { value: W } | L1 power derived from V×I |
| /Ac/L1/Voltage | { value: V } | L1 voltage (V) |
| /Ac/L1/Current | { value: A } | L1 current (A) |
| /Ac/L2/Power | { value: W } | L2 power derived from V×I |
| /Ac/L2/Voltage | { value: V } | L2 voltage (V) |
| /Ac/L2/Current | { value: A } | L2 current (A) |
| /Ac/L3/Power | { value: W } | L3 power derived from V×I |
| /Ac/L3/Voltage | { value: V } | L3 voltage (V) |
| /Ac/L3/Current | { value: A } | L3 current (A) |
| /Current | { value: A } | Actual charging current setpoint |
| /MaxCurrent | { value: 32 } | Hardware maximum (fixed at 32A) |
| /Ac/Energy/Forward | { value: kWh } | Total energy charged |
| /CustomName | { value: string } | Display name in VRM |

Base topic: venus-home/W/<venus_id>/evcharger/{{ instance }}

## 6.3  Bidirectional topics (Victron → HA)

| Topic (as seen by HA) | Description |
| --- | --- |
| venus-home/N/<venus_id>/evcharger/+/SetCurrent | Victron/Node-RED commands a new current (A) |
| venus-home/N/<venus_id>/evcharger/+/StartStop | Victron/Node-RED commands start (1) or stop (0) |
| venus-home/N/<venus_id>/evcharger/+/Mode | Victron mode: 0=Manual 1=Auto 2=Scheduled |

## 6.4  Node-RED internal topics (Cerbo broker, no prefix)

| Topic | Description |
| --- | --- |
| N/<venus_id>/evcharger/+/Ac/Power | EV charger current power (read by Node-RED) |
| N/<venus_id>/evcharger/+/Status | EV charger status (read by Node-RED) |
| N/<venus_id>/evcharger/+/Mode | EV charger mode from Victron (read by Node-RED) |
| W/<venus_id>/evcharger/<n>/SetCurrent | Node-RED writes target current |
| W/<venus_id>/evcharger/<n>/StartStop | Node-RED writes start/stop command |

# 7. Troubleshooting

## 7.1  Diagnostic commands

Run these on the Cerbo via SSH (ssh root@<cerbo-ip>):

# Check dbus-mqtt-devices is running

svstat /service/dbus-mqtt-devices

# Watch dbus-mqtt-devices log

tail -f /var/log/dbus-mqtt-devices/current ¦ tai64nlocal

# Check registration arrives on Cerbo

mosquitto_sub -h 127.0.0.1 -p 1883 -t 'device/#' -v

# Watch write topics arriving from HA

mosquitto_sub -h 127.0.0.1 -p 1883 -t 'W/#' -v

# Watch all EV charger topics

mosquitto_sub -h 127.0.0.1 -p 1883 -t 'N/<venus_id>/evcharger/#' -v

# Check EV charger on D-Bus

dbus-spy

## 7.2  Common issues

| Symptom | Cause | Fix |
| --- | --- | --- |
| EV charger not visible in VRM | dbus-mqtt-devices not registered | Check registration topic arrives on Cerbo. Check driver is running with svstat. |
| DBus response never arrives in HA | Bridge direction missing | Confirm topic # both 0 venus-home/ is in bridge config and Mosquitto restarted. |
| Input helpers stay empty | DBus trigger topic wrong | Trigger should be venus-home/device/ha_bridge/DBus not without prefix. |
| Node-RED shows Waiting for instance | EV_INSTANCE not set | Wait for HA to publish registration and for DBus response. Check fn-instance status. |
| Node-RED inactive despite Auto mode | ev_mode not populated | Check mqtt-ev-mode node — ensure topic N/<venus_id>/evcharger/+/Mode receives data. |
| Charger stops on every ramp down | MAX_RAMP_DOWN too high | Feyree rejects drops > 2A. Keep MAX_RAMP_DOWN at 2. |
| SetCurrent not applying to Feyree | Bidirectional automation condition | Check trigger.topic.split('/')[4] matches the stored instance number. |
| Power shows as 0W in VRM | Status entity name mismatch | Verify sensor entity names in tuya-local match exactly what the automation uses. |
| Numeric values sent as strings | Payload formatting error | Ensure JSON payloads produce numbers not strings: {value: 21.5} not {value: "21.5"}. |

## 7.3  Verify end-to-end with a manual test

- In HA Developer Tools → MQTT, publish to venus-home/device/ha_bridge/Status with the registration payload

- Within 5s, subscribe to venus-home/device/ha_bridge/DBus — you should see the instance numbers

- Check the input_text helpers populated

- Trigger the push automation manually — check the W/ topics arrive on the Cerbo (mosquitto_sub)

- Check VRM → your site → EV charger appears in device list

- In VRM, set mode to Auto — watch Node-RED fn-calc status change to active

# 8. Deployment Checklist

| Step | Action | Done |
| --- | --- | --- |
| 1 | dbus-mqtt-devices v0.9.0 installed on Cerbo — svstat shows running | ☐ |
| 2 | MQTT on LAN (Plaintext) enabled on Cerbo (Settings → Services) | ☐ |
| 3 | Mosquitto bridge config saved and add-on restarted | ☐ |
| 4 | 5 input_text helpers created in HA | ☐ |
| 5 | tuya-local detects Feyree — all sensor entities visible in HA | ☐ |
| 6 | Registration automation deployed and triggered manually | ☐ |
| 7 | input_text.victron_instance_ev1 populated (non-empty) | ☐ |
| 8 | Push automation deployed and triggered — W/ topics seen on Cerbo | ☐ |
| 9 | EV charger visible in VRM device list | ☐ |
| 10 | Bidirectional automation deployed | ☐ |
| 11 | Node-RED ev-surplus-flow.json imported and deployed | ☐ |
| 12 | All 5 Victron input nodes confirmed in Node-RED | ☐ |
| 13 | CONFIG node parameters verified for your hardware | ☐ |
| 14 | VRM mode set to Auto — fn-calc shows active status | ☐ |
| 15 | First 30s tick fires — SetCurrent/StartStop published | ☐ |
| 16 | Manual test: change VRM mode to Manual — Node-RED goes inactive | ☐ |

# 9. References

- dbus-mqtt-devices source: https://github.com/freakent/dbus-mqtt-devices

- Victron D-Bus EV charger paths: https://github.com/victronenergy/venus/wiki/dbus#evcharger

- tuya-local HACS integration: https://github.com/make-all/tuya-local

- Venus OS Large (Node-RED): https://www.victronenergy.com/live/venus-os:large

- Victron VRM portal: https://vrm.victronenergy.com

- Venus OS root access: https://www.victronenergy.com/live/ccgx:root_access

- Feyree EV charger HA community thread: https://community.home-assistant.io/t/feyree-wifi-smart-ev-charger/718634